Creating a Miboard

What you will need.

A microcontroller board minimal requirement arduino Mega 2560  based board or just a arduino Mega 2560

A lot of breadboard jumper wires 65PCS and a female to male dupoint wires 20PCS

A LCD 1602 Module

a 4x4 keypad

A computer for uploading code

A usb cable for uploading

A 830 tie-points breadboard

A potentiometer 10K 1 PCS

A power supply for the board

A DS1307-Module-V03   as the RTC or Real Time Clock

We would recommend a elegoo kit with a mega 2560.

Creating the Miboard

Now that we have done that lets attach the keypad , RTC and LCD

you can use the diagram image to help you wire up the stuff

here is a pinout of the keypad :

The First keypad pin starts at the side were we find the star button

keypad pin 1 = A0

keypad pin 2 = A1

keypad pin 3 = A2

keypad pin 4 = A3

keypad pin 5 = A4

keypad pin 6 = A5

keypad pin 7 = PWM 3

keypad pin 8 = PWM 2

The pinout for the RTC is :

GND = GND

VCC = Positive connection on LCD breadboard

SDA  = SDA 20

SCL = SCL 21

We don't need the SQW pin

To connect the LCD you will need to use the picture above.

Uploading the code

To upload the code enter the arduino IDE.

Then if you are using windows or mac please download the git application if you are using linux download git using the package manager

then if using windows or mac launch the git terminal otherwise if you have linux just open a terminal.

Now type this command:

git clone https://github.com/meowgle/MOSM.git

go to the home folder and then go to the MOSM folder and upload the MOSM.ino

Or just go to the github page and download it there:

https://github.com/meowgle/MOSM

then power it on and you have yourself a MIboard a little small computer for typing numbers and adding them by counting them

and telling time.

just a device that you can tinker with

there is also source code to the kernel that is used in MIboardOS or MOSM.

click here to download the libraries :

https://github.com/meowgle/MOSM/tree/main/libraries

In the future we might make it a single board computer but for now it's just a arduino project.

but the single board computer will only have a microcontroller no ARM processor .